Technical Specification for Procurement of 34 nos. Garbage Dump Truck (3 Ton)

| Item No. | Name of Goods | Detailed Technical Specification and Standards required | Full Detailed Technical Specifications and Standards offered |
| --- | --- | --- | --- |
| 1 | Garbage Dump Truck (3 Ton) [General Requirement] | The proposed Dump Truck of 4 WD (Four Wheel Drive) shall have a carrying capacity of minimum 3 tons and shall be used mainly for transporting household solid waste/municipal compost having corrosive properties. The Dump Truck shall be brand new and consist of an all-steel open garbage load body mounted on a truck chassis. The load body is to be operated by a hydraulic hoist which shall be activated by a power take off (P.T.O.) system. Loading of garbage on load body can be manual, but unloading is done by operating the hydraulic cylinder inclining load body at the rear with hydraulic/electric control panel installed inside the cabin of truck. The back door/tailgate will open automatically while unloading the garbage and will close automatically when not working. The side will not leak water and also the back door will not leak water by adding a sloping angle at the tail of the dump body, as such the water will be kept inside the container of the truck. Any other distinct functional features that are not mentioned above, the Bidders are requested to elaborate in their offer, supported by the original catalogue. Equipment Brand: To be mentioned by the bidder Equipment Model: To be mentioned by the bidder Country of Origin: To be mentioned by the bidder Year of manufacturing: To be mentioned by the bidder (Not before 2026) |  |
| 2 | Garbage Dump Truck (3 Ton) [Chassis and Engine] | A diesel engine having minimum 4 cylinder, Inline, 4 (four) cycles naturally aspirated or turbo diesel, direct injection type engine. (i)Chassis Brand: To be mentioned by the bidder (ii)Engine Brand: To be mentioned by the bidder (iii)Original Equipment Manufacturer (OEM) and chassis manufacturer should be the same. The bidder will specify the following in its offer supported by original brochure: (a) Displacement volume: Minimum 3200 cc (b) Engine Type: To be mentioned by the bidder (c) Engine Model: To be mentioned by the bidder (d)Torque: Minimum 350 Nm at rated rpm (Corresponding Engine performance curve should be attached) (e) Power: Minimum 85 kW at rated rpm (Corresponding Engine performance curve should be attached) (f) Fuel system: Direct injection (g) Fuel Tank: Min. 100 liters (h)Engine Emission: TIER II/TIER III or equivalent. (emission standard certificate from the appropriate authority should be provided with the bid) |  |
| 3 | Garbage Dump Truck (3 Ton) [Transmission] | Manually operated, Minimum 5 (five) forward and 1 (one) reverse synchromesh gears. |  |
| 4 | Garbage Dump Truck (3 Ton) [Clutch] | Hydraulic control, dry single plate with diaphragm spring. |  |
| 5 | Garbage Dump Truck (3 Ton) [Suspension (front & rear)] | Semi-elliptical, alloy steel leaf springs with hydraulic double-acting telescopic shock absorber. |  |
| 6 | Garbage Dump Truck (3 Ton) [Steering] | Right Hand Drives having power assisted steering with a tilt & telescopic steering column. |  |
| 7 | Garbage Dump Truck (3 Ton) [Service brake] | Hydraulic dual circuit, front: 2-leading, rear: dual 2-leading Parking brake: Mechanical expanded type at rear of transmission |  |
| 8 | Garbage Dump Truck (3 Ton) [Cabin] | i) Standard tilt type cabin, ii) Minimum 2 (two) seats including driver. |  |
| 9 | Garbage Dump Truck (3 Ton) [Axle &Tire] | 2 (Two) axle, 6 tires, 1 spare tire. Tire size (front & rear): To be mentioned by the bidder |  |
| 10 | Garbage Dump Truck (3 Ton) [Electrical System] | 12V/24V, 60A. |  |
| 11 | Garbage Dump Truck (3 Ton) [Vehicle weight carrying capacity] | ii) Carrying capacity (payload): Minimum 3000 kg ii) Gross vehicle weight: To be mentioned by the bidder iii) Curb weight: to be mentioned by the bidder |  |
| 12 | Garbage Dump Truck (3 Ton) [Dimensions ] | Wheel base: Minimum 3000 mm; Overall length: Minimum 5000 mm; Overall width: Minimum 1800 mm; Overall height: Minimum 2100 mm; Ground clearance: Minimum 180 mm |  |
| 13 | Garbage Dump Truck (3 Ton) [Dump Body] | The internal length and width shall be according to the coverall length and width: Internal Body length: Minimum 3500 mm; Internal Body width: Minimum 1600 mm; Internal Body height: Minimum 500 mm (inner); Thickness of deck floor: Minimum 3 mm MS + Minimum 1.5mm SS lining; Thickness of wall: Minimum 3 mm MS+ Minimum 1.5 mm SS lining; Tipping angle: Minimum 45 degrees; Turning Radius: 8.5 m (maximum); Raising and lowering time: 30 seconds (maximum) |  |
| 14 | Garbage Dump Truck (3 Ton) [Accessories Tools] | Accessories to be supplied with each unit: 1. Speedometer (km/hr.) with odometer 2. Engine oil pressure gauge or warning light 3. Fuel level indicator 4. Water (engine) temperature indicator 5. Head, cabin lights, reversing light & traffic indicator etc. 6. Heavy-duty battery 7. Horn 8. Spare wheel 9. 1 (one) Jack (Minimum 3 Ton), one set wheel wrenches & One Grease Gun. 10. Manufacturer Standard Tool kits 11. Rear-view mirrors (2 side, 1 interior) 12. Reversing alarm/beeper stem |  |
| 15 | Garbage Dump Truck (3 Ton) [Spare parts] | Bidder shall provide a manufacturer's certificate ensuring that spare parts will be made available in the purchaser's country for a period of at least 05 (five) years from the date of supply. The bidder shall supply the following spare parts for each dump truck. (The value of listed mandatory spare parts will be taken into account during the bid evaluation of the supply items.) i)Fuel Filter :15 Nos ii)Air Filter :15 Nos iii)Oil Filter:15 Nos iv)Hydraulic Filter:15 Nos |  |
| 16 | Garbage Dump Truck (3 Ton) [O & M Training] | Minimum 5 officials will be nominated for O&M training preferably before the pre shipment inspection at manufacturer premises. O & M training of one driver & one Sub assistant engineer for each equipment nominated by the purchaser for 1 (One) working day at the work site within 7 (Seven) days of delivery. The cost of these services shall be included in the bid price but not paid as a separate item. |  |
| 17 | Garbage Dump Truck (3 Ton) [After Sales Service] | Bidder must have to declare the after sales service of equipment related any issues during the warranty period and must have to submit the documents of assigned workshop in Bangladesh with all details of workshop accessories along with regarding personnel. |  |
| 18 | Garbage Dump Truck (3 Ton) [Documents and Tools to be submitted] | During bid submission a) original catalogue / Brochure (in English), supportive of the offered equipment model and respective technical specifications; b) Certificate assuring a minimum of 5 years after-sales service facility in the purchaser's country with detailed capacity (list of technical persons and workshop equipment) of the service center; c) List of manufacturer's standard tools; During delivery: a) Maintenance manual in English (Soft or Hard copy) b) Spare Parts catalogue in English (Soft or Hard copy) c) Engine manual in English (Soft or Hard copy) d) Manufacturer's standard tools |  |
| 19 | Garbage Dump Truck (3 Ton) [Warranty] | The warranty requirement is minimum 3 (three) Years from the final acceptance of goods. |  |